Electrostatics & Energy:

- Matter – The substance that comprises all living objects

  - Principle characteristic = mass/weight

- Energy - The ability to do work

  - Principle characteristic is movement/motion

  - Work – The result of force acting on an object over a distance

  - Power – incorporates time into equation

    - Same amount of work to lift puppy to certain height (5 mins or 5 seconds)

    - More power lifts quicker

    - Many types energy – mechanical, chemical, nuclear, electromagnetic, electrical

- Combination of matter & energy is a constant

  - They can convert to each other but can’t be destroyed

- Energy:

  - Electrical energy –

    - Can be converted:

      - Light bulb – electrical to radiant/light

      - Hair dryer – electrical to thermal

  - Electromagnetic spectrum

    - Electromagnetic radiation/energy placed on a continuum based on type of electrical excitation

    - Rainbow is a small portion in the middle

    - Visible light in the middle

    - Ultraviolet – above violet on the spectrum

    - Infrared – below red

    - Human eye can see very little

    - Electromagnetic waves can travel through a vacuum without losing energy

      - Vacuum – space entirely devoid of matter

        - Semi-vacuum x-ray tube used in radiology

  - Particle Wave Theory

    - Energy described as a wave – high, low, frequent, infrequent

    - Waves:

      - Measurable height & frequency

      - Wavelength is distance between wave crests

        - Shorter wavelength = higher frequency

      - Crest – wave highpoint

      - Trough – wave low point

  - Frequency - Rapidity with which waves hit the shore (symbol is hertz/Hz)

    - Period – Time taken to complete one whole wave

    - Cycle – One crest & trough

    - Amplitude – height of wave from crest to trough

    - Sine wave – tracing of waves as they travel

      - Have a high and low point with time as a constant

      - Important in choosing radiology time settings

    - Sound waves – travel invisibly through space

  - Energy in Radiology

    - Energy is thought of as particles (x-rays or gamma rays)

      - Mathematic quantification of amount of energy needed to do work

    - Photon particle carries a specific energy that depends on frequency

      - Energy & frequency are directly proportional

        - Energy doubled = frequency doubled

  - Roentgen’s Properties of x-rays – still the original list

    - Won Nobel Prize in Physics in 1901 for producing/detecting x-rays

    - X-Ray Properties – Very important

      - Highly penetrating invisible rays that are a form of electromagnetic radiation

      - Electrically neutral – not affected by electric or magnetic fields

      - Can be produced over a wide variety of energies & wavelengths

      - Release small amount heat as pass through matter

      - Travel in straight lines

      - Travel at speed of light – 3 X 108 meters/sec in vacuum

      - Can ionize matter

      - Cause fluorescence (light emission) of certain crystals

      - Cannot be focused by a lens

      - Affect photographic film

      - Use ionization & excitation to produce chemical/biological changes

      - Produce secondary & scatter radiation

Magnetism & Electricity

- Electricity becomes x-rays

  - Atoms:

    - Protons in nucleus positive

      - Tightly bound

    - Electrons orbiting are negative

      - Vulnerable to outside forces

        - Charges can be weak

        - Bonds can be broken

      - Electrons can be cast off

        - If not replaced, atom stays positively charged

        - Can float freely (free electrons) or join other atoms

    - Strives for balance of pos/neg

    - Electricity concerns the movement of the electrons

  - Electrostatics

    - Static electricity/electrostatic charge – Radiant energy

      - Charge of free electrons builds up & dissipates when an object that will accept a charge (conductor) is encountered

        - Static electricity in hair –

          - Hairs build up excess electrons so are negatively charged & repel each other

        - Shocking others – Feet scoop up extra electrons, then discharge and lose them when touches something that conducts

        - Cold, dry atmospheres enhance build-up

          - Wetting a comb allows electrons to redistribute appropriately

        - Radiation works this way on a greater scale

      - Electrification – The process of electron charges being added/subtracted from an object

  - Laws of electrostatics – Very Important

    - Electricity called packets of electron energy

    - 5 laws:

      - Like charges repel; unlike attract

        - Fundamental basis of magnetism

        - Principle of an electric circuit

        - How an x-ray tube prepares to make an exposure

      - Inverse square law  [Flashlight]

        - One of the most important concepts in the field of imaging

        - Impacts set-up of x-ray unit & technical factors

        - The intensity of the x-ray beam is inversely proportional to the square of the distance from the source

          - Moving tube closer to subject intensifies the radiation

            - And illumination

          - ½ distance = radiation X 4

      - Distribution – The charges reside on the outside surfaces of conductors but all through nonconductors

      - Concentration

        - Greatest concentration of charges is on surface where curvature is steepest

        - Not as applicable in radiology but  governs voltage from taser or cattle prod

      - Movement – Only negative charges move along solid conductors

        - Positive charges tightly bound

  - Electrification – Object electrified in 3 ways

    - Contact –

      - Shocking a friend

        - Electrons collect on body surface, giving you negative charge

        - Electrons concentrate on point of finger

        - Finger close enough – spark discharges into electrical neutral or slightly pos

      - Static electricity can build up on an x-ray tray in a very dry atmosphere

        - Film records exact amount of discharge

    - Friction – Occurs when one object rubs against another

      - Electrons travel based on availability of electrons in each object

    - Induction – Most important for radiology

      - Electrical fields act upon each other without actual contact

      - Uses the force fields of the electrons of one object  to cause a reaction in an opposing object

      - Principle directly applies to x-ray production

        - Also transformers, x-ray tubes, electric motors

      - Lightning:

        - Clouds build up excess electrons when form thunderheads

        - Ground is neutral

        - Discharge of electrons from cloud to ground is instant, loud

        - Electric current is generated between cloud & ground

  - Conductors & Insulators

    - Some objects conduct easily

      - Water – no appliances in bath

      - Copper – Used in household wiring

    - Some don’t conduct

      - Rubber, plastic, and glass

      - Rubber covering over copper wire

Electric Current:

- Andre-Marie Ampere – described electric current as a quantity of electrons flowing past a pt in time

  - Ampere is unit of current

  - Milliamperes (mA’s) –

    - Diagnostic imaging uses these to regulate # of electrons to produce x-ray photons

    - mA setting determines number of electrons to flow past a given point

      - Increasing mA will increase image density or darken

      - Decreasing mA will lighten

      - Doubling mA’s doubles density

- Resistance – the opposite of current flow

  - George Ohm researched resistance & ohm is the unit of resistance

- Potential Difference:

  - Causes electrons to travel from one end of the wire to then other

  - Empty Garden hose –

    - Little water in hose but lots of potential in faucet

    - Faucet turned on to allow flow - potential difference along hose

      - One end with none & one end has lots of water coming in

    - Potential difference is measurement of empty hose compared to full

  - Diameter impacts

    - Small hose/wire has lots of resistance

    - Larger hose/wire has little

  - Voltage

    - Alessandro Volta – identified unit of potential difference

      - Electromotive force – Force that draws electrons from area of excess to area deficient of electrons in circuit

      - Volt - Strength of potential difference or strength of electron flow

Diagnostic X-ray production

- Electrical circuit requires:

  - Potential difference (voltage)

  - Resistance (ohms)

  - Amperage (current)

- Power

  - Watt – unit of power

    - Named after James Watt

    - Colleagues with Ampere, Volta, and Ohm – aware of work

    - 1 watt = 1 ampere flowing through a circuit at 1 volt per second

    - Current in amperes (I)  X volts (V) = Power (W or watts)

      - I X V = W

      - Example – Most x-ray units use a maximum of 125 volts & 300 milliamperes of electricity

        - W = 0.3 X 125,000 = 37,500 watts or 37.5 kilowatts

        - Most generators are rated at a minimum of 30 KW’s

        - So should be OK

- Electrical circuit

  - Power originates at a power plant & is transmitted with periodic transformer stations that boost power

  - The transformer nearest the clinic is most important

    - May be an auxiliary just outside clinic

  - Closing a switch completes the circle or circuit

    - Connection is needed to turn the system on/off

    - Breaking the circuit causes the light to go off

      - Burnt out light bulb:

        - Fine wire the forms the cathode of the lamp burns out in the heat of the electron flow

        - Circuit is interrupted and light doesn’t function

      - Mouse eats through cord:

        - As cord is eaten through – mouse conducts electricity & dies

  - On-off switch – On all units, plus

    - Wall switch – usually required by law to be at eye level within reach of the generator

      - Power can be shut off as alternate means

    - Line Voltage Compensator – standard equipment

      - Usually automatic on newer units

      - Older ones have compensator mounted on control panel

      - Means for increasing/decreasing incoming line voltage

  - Circuit Breakers

    - Power supply to x-ray unit

    - Accepts current (amperage) up to its limit/rating

    - When hits limit – circuit breaker disconnects & power is interrupted

    - Fuse works the same way except a small piece of metal melts to stop flow

    - Important factors to circuit:

      - Current/amperage

        - If too much current is demanded by generator – overload light comes on and no exposure is possible

        - Current in generator is milliamperage (mA’s)

    - Very important – should never be bypassed

  - Ground

    - Circuit must be grounded to be safe

    - Needs an alternate route for electricity to flow if circuit breaks inappropriately

    - Original circuit is directed to a ground wire attached to an object that absorbs electrons

    - Flow of electrons stops but excess go to ground wire

  - Direct vs alternating current

    - Direct – batteries installed in a tube/line (flashlight, car battery)

      - Orientation matters

      - Requires source situated close to end user

      - If energy needed to go large distances – need low current/high voltage (more efficient)

    - Alternating current – produces 1 pos and 1 negative pulse (cycle)

      - Most efficient way to transmit power = 100 cycles per second

      - Requires a transformer to step up power at one end and down at other

      - Heinrich Hertz – studied electromagnetism

        - Hertz is # cycles per second of oscillations of pos/neg

        - 1 Hertz = 1 pos and 1 neg cycle

  - Transformers –

    - Receive power from incoming power lines & transform power to x-ray tube

    - Use turns of wire around a central core or two close cores

    - Increase or decrease voltage in a circuit

    - Uses induction to electrify

    - 3 transformers in the x-ray circuit:

      - Autotransformer

        - Increases incoming voltage

        - Kilovoltage selector – allows selection of KW’s to produce radiograph

        - Central core & taps

      - High voltage transformer – very powerful

        - Final step-up transformer to boost voltage to x-ray tube

        - Raises voltage from incoming 220 to maximum 125,000 volts

      - Filament transformer

        - Smaller, step-down transformer

        - Produces voltage to the filament of the x-ray tube

          - Tube works like light bulb

          - Small wire visible in bulb is cathode

            - Produces light depending on wattage rating

            - Produces electrons in cloud governed by selection factors

            - Filament must reach certain temp for exposure to take place

            - Filament produces this temp

      - Rectifiers

        - Changes current flow on negative part of AC cycle to positive so entire flow can travel to x-ray tube to produce radiation

          - If only half the current available, only half radiation produced

        - X-ray tube can only receive positive charges, since current only can flow one way

        - Additional feature added to ensure each exposure starts at beginning of pulse and finished at end – all current can be used

      - Three-phase circuits

        - Add in 2 more pulses of power offset from 1st pulse to make the dips in power in the waveform less significant

        - Requires a lot more power

        - Only used in large hospitals needing to image large chests/abdomens

      - High frequency

        - Uses high-frequency pulses to keep ripple low with continuous output

        - More x-rays per exposure reduces overall exposures

          - Saves wear & tear on unit & reduces overall dose

- Outside of X-ray unit

  - 3 items essential to every unit

    - Control panel/generator – sometimes mounted above table

    - X-ray tube

    - High-tension transformer

  - Unit requires dedicated power line

- Large Animal portable x-ray units

  - Smaller version – not as powerful

  - Less milliamperage

  - Usually only needed for legs & feet

  - Generator, transformer, and x-ray tube compressed

Diagnostic x-ray production

- All in place:

  - X-ray circuit receives incoming power

  - Step-up transformers supply power to cathode side of x-ray tube

    - Cathode usually has 2 filaments made of thoriated tungsten

    - Can withstand high temps without melting

    - Electrons are boiled off cathode filament in reaction called thermionic emission

    - Cloud of emissions called a space charge

    - Whole process is space charge effect

    - Has a large & small focal point

  - Filament circuit heats cathode to high temp (sometimes 3900 degrees F)

    - Which filament is used is determined by mA setting

    - Filaments positioned in cup-shaped focusing cup so aimed directly at anode

    - Focusing cups slightly negatively charged to focus electrons boiled off cathode filaments

    - Beveled edges also help focus

    - Focal spot of anode is where x-rays are produced & where most heat must be dissipated

  - X-ray tube receives power, produces electrons, and converts them to x-rays

    - Produces extreme heat

    - Differences in tubes have to do with cooling systems

      - Ratio of heat to x-rays is 99-1

      - Method of heat transfer enhances tube longevity & reliability

    - Tube consists of glass enclosure that houses specialized anode & cathode

      - Glass enclosure is heat-resistant (sometimes Pyrex)

      - Standard tube is 30 cm long & about 20 cm diameter

  - Anode

    - 2 types:

      - Rotating – small animal units

      - Stationary – large animal units

    - Usually made of tungsten or an alloy – high atomic number so can absorb electrons/heat

    - Target of the tube is mounted in rotating anode – prepared to accept electrons

    - Functions:

      - Mechanically supports electron target

      - Serves as thermal dissipater by directing heat emitted in x-ray production

      - Rotates so photons aren’t always focused in same spot

      - Electrical conductor – receives electrons & transmits them back

    - Rotor circuit

      - Activated when the filament transformer starts to heat the cathode

      - Rotor turns the rotating anode

      - Contains stainless steel ball bearings to withstand high temperatures

        - Bearings can distort as x-ray tube ages

        - Increases noise since move rapidly when rotor is turning

        - If bearings seize, rotor doesn’t turn, and the heat stays fixed on focal spot

        - Anode overheats & cracks, safety interlocks kick in

    - Stationary anode

      - Used in equine portable units

      - Needs a way of dissipating heat, absorbing photons, and converting to x-rays

      - Anode made of copper w/a tungsten insert

      - End is angled to direct the beam to the patient

      - Does not rotate so must watch warning/ready lights

  - The Line Focus Principle

    - Ensures the x-rays are directed onto object being radiographed

    - Describes how the electrons interact with the anode & change direction so x-rays are directed towards patient

    - Line focus principle – Determines the width of the beam & resolution

      - Involves the angle of the bevel on outer edge of anode & resulting change in direction of x-rays

      - Angle < 15 degrees from vertical = beam narrow with high resolution

      - Angle > 15 degrees = beam wider, less heat focused, decreased bloom effect

      - Typical angle is 11 degrees

  - Off-focus radiation

    - Exposure switch is closed = electromagnetic function of circuit takes effect

    - Cloud of negatively charged particles drawn across to positively charged anode

    - Electrons interact with target + areas of anode adjacent to target

      - Electrons can bounce off the target, then be attracted back beyond the focal point

      - Collar of lead around tube normally prevents extra-focal radiation

      - Can appear as artifact

  - Heat Bloom

    - Target exposed to radiation = anode can heat to 1000-2000 degrees

    - Heat bloom - Repeated exposures can cause heat dissipation & enlargement of focal spot

    - Usually not enough exposures in vet clinic to happen

  - Tube rating chart

    - Indicates the x-ray tube limits based on the heat units equation

      - mA X kV X time

      - Determines suitability for size of animals clinic radiographs

  - Focal spot bloom

    - Affects the sharpness of the image

    - Can happen with old tube

    - Anode bombarded with radiation gets hot

    - Heat dissipates into surrounding metal with time

    - If tube not allowed to cool, outer edges of focal spot become hot enough to expand spot

    - Causes image to lose sharpness, reducing resolution

      - Increased focal spot size causes unsharpness

  - Anode heel effect

    - Bevel of the anode limits the amount of x-rays produced on stem side of anode

    - Intensity of radiation is greater on cathode side

    - Important when pt is thicker at one end than the other

    - Thicker end should go at cathode side

      - Head end to right, foot to the left

  - Exposure switch

    - Unit that sets the sequence of events in motion to produce the x-ray exposure

    - Usually 2-stage

      - Wait for the ready signal (especially in mobile units)

        - Prolongs lie of unit

      - 1st activates the rotor & boosts the filament and transformers

        - Rotor noise should be evident

        - If rotor doesn’t activate, don’t expose since tube can be damaged

          - Exposure could be directed to very small focal point

          - Will eventually melt the anode

        - If hear boiling liquid – stop

          - X-ray tube likely shorted out and filament circuit is overeating the cooling oil in the x-ray tube housing

          - Tube cold explode if exposure is attempted

      - 2nd activates the exposure through the x-ray tube

      - Be familiar with noises

      - “Dead man” safety factor - Test at least annually to make sure disconnects when pedal released

        - Legal requirement to make sure x-ray beam terminates

    - Exposure switch variations

      - Single-stage – wired so rotor begins when generator turned on

        - Not ideal, should be activated only when exposure is about to be made

      - Single-stage foot switch – initiates rotor when depressed

        - Safety delay until unit picks up speed

        - Lessens flexibility in positioning fractious animal

      - Hand switch – most human units

        - Usually behind wall

        - Replace with foot for veterinary